Implement an SDN-based Traffic Monitoring and Statistics

Collector using Mininet and Ryu controller that periodically

retrieves flow statistics and displays packet/byte counts

per flow.

- 1 OpenFlow Switch (s1)

- 4 Hosts (h1, h2, h3, h4)

- Remote Ryu Controller on 127.0.0.1:6633

1. Start Ryu controller:

source ~/ryv-env/bin/activate

ryu-manager traffic_monitor.py --ofp-tcp-listen-port 6633

2. Start Mininet (new terminal):

sudo mn --controller=remote,ip=127.0.0.1,port=6633 --topo single,4

3. Run test scenarios in mininet> CLI

Scenario 1 — Connectivity test:

mininet> pingall

Scenario 2 — Bulk traffic measurement:

mininet> iperf h1 h3

Expected: High throughput bandwidth measurement between h1 and h3

## Expected Output

- Flow stats table printed in Ryu terminal every 5 seconds

- Packet and byte counts increase with traffic

- traffic_report.log file generated with timestamped flow data

## Results

- pingall: 0% dropped (12/12 received)

- iperf: 47.6 Gbits/sec, 55.5 GBytes transferred

- Flow stats collected and logged successfully

## References

- https://mininet.org/overview/

- https://ryu.readthedocs.io/en/latest/

- https://github.com/osrg/ryu